Pseudo-Code: Overall Simulation and PCI Optimization Flow

Pseudo-Code 1: Main Simulation Workflow

Input:
    Network topology parameters
    Number of eNodeBs and sectors
    UE mobility and traffic models
    PCI optimization algorithm (ILP / DASTUR / MP-BRKGA)

Output:
    Optimized PCI assignment
    SINR and interference statistics

Begin
    Initialize NS-3 simulation environment
    Configure LTE parameters (bandwidth, frequency, scheduler)
    Deploy eNodeBs and sector antennas
    Deploy UEs and assign mobility models

    Run baseline simulation with random PCI allocation
    Collect SINR, RSRP, and handover statistics

    Build interference graph from simulation measurements
    Execute selected PCI optimization algorithm
    Obtain optimized PCI assignment

    Apply optimized PCI configuration to the network
    Run post-optimization simulation
    Collect SINR and interference statistics

    Compare baseline and optimized results
    Export performance metrics and plots
End

Pseudo-Code 2: Interference Graph Construction

Input:
    SINR measurements
    Neighbor relations (ANR)
    Thresholds for interference detection

Output:
    Weighted interference graph G(V, E)

Begin
    Initialize graph G with one vertex per cell
    For each pair of neighboring cells (i, j):
        Compute average SINR degradation
        If SINR < predefined threshold:
            Add edge (i, j) to G
            Assign weight based on interference severity
    EndFor
End

Pseudo-Code 3: DASTUR Algorithm (Iterative Heuristic)

Input:
    Interference graph G(V, E)
    PCI pool

Output:
    PCI assignment minimizing conflicts

Begin
    Initialize PCI assignment randomly
    Compute initial interference cost

    Repeat
        For each cell v in V:
            Evaluate alternative PCI values
            Select PCI that minimizes local interference
        EndFor

        Compute new global interference cost
    Until no further improvement is achieved

    Return best PCI assignment
End

Pseudo-Code 4: MP-BRKGA Algorithm

Input:
    Interference graph G(V, E)
    Population size P
    Maximum generations Gmax

Output:
    Near-optimal PCI assignment

Begin
    Initialize population with random keys
    Decode each individual into PCI assignment
    Evaluate fitness based on interference cost

    For generation = 1 to Gmax:
        Select elite individuals
        Generate offspring via biased crossover
        Apply mutation with predefined probability
        Decode and evaluate new individuals
        Update population
    EndFor

    Return best individual found
End

Pseudo-Code 5: ILP Formulation (High-Level)

Decision Variables:
    x(i, p) = 1 if PCI p is assigned to cell i, 0 otherwise

Objective:
    Minimize sum of interference weights for conflicting PCIs

Constraints:
    Each cell must be assigned exactly one PCI
    Neighboring cells cannot share the same PCI
End