Input: ServingCellID, UE_Measurements_Map, Base_PCI_Penalty_dB, Scaling_Factor α
Output: Penalty_Factor and updated UE Noise Figure

If ServingCellID == 0 or UE_Measurements_Map is empty then
    return Penalty_Factor = 1.0
End If

Serving_RSRP_dBm ← Average RSRP of ServingCellID
Serving_Power ← Convert_dBm_to_Linear(Serving_RSRP_dBm)
Total_Penalty_dB ← 0

For each MeasuredCell in UE_Measurements_Map do
    If MeasuredCell.ID ≠ ServingCellID and
       (MeasuredCell.PCI mod 3 = ServingCellID mod 3) then

        Interferer_RSRP_dBm ← Average RSRP of MeasuredCell
        Interferer_Power ← Convert_dBm_to_Linear(Interferer_RSRP_dBm)
        ISR ← Interferer_Power / Serving_Power

        If ISR > 0 then
            Dynamic_Penalty_dB ← Base_PCI_Penalty_dB + α × (10 × ISR)
            Total_Penalty_dB ← Total_Penalty_dB + Dynamic_Penalty_dB
        End If
    End If
End For

Update UE Noise Figure ← Base_Noise_Figure + Total_Penalty_dB

If Total_Penalty_dB > 0 then
    Penalty_Factor ← 10^(Total_Penalty_dB / 10)
Else
    Penalty_Factor ← 1.0
End If

Use Penalty_Factor during CQI reporting